- HD-SFP+/10G-ZR

- 10.3Gbps SFP+ Transceiver, Single Mode, 80km Reach

Features

	Hot-pluggable SFP footprint

	Duplex LC fiber connector

Cooled EML transmitter and APD receiver

	Transmission distance up to 80Km on 9/125µm SMF

	Built-in DDMI functions

	Comply with SFP MSA .

	Single +3.3V power supply

	RoHS6 compliant (lead free)

	Operating case temperature:

Commercial :   0 to +70°C

Industrial :     -40 to +85°C

	Comply with Ethernet Switches from a lot of widely used brands .

Applications

	10GBASE Ethernet

	SONET OC-192 SDH STM-64

	Switch to Switch interface

	Router/Server interface

	Other optical transmission systems

Optical Characteristics

| Parameter |  | Symbol |  | Min | Typical | Max | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |  |  |
| Centre Wavelength |  |  | λc | 1530 | 1550 | 1565 | nm |  |
| Spectral Width（-20dB） |  |  | Δλ |  |  | 1 | nm |  |
| Side-Mode Suppression Ratio |  |  | SMSR | 30 | - |  | dB |  |
| Average Output Power |  |  | Pout | 0 |  | +4 | dBm | 1 |
| Extinction Ratio |  |  | ER | 6.0 |  |  | dB |  |
| Data Input Swing Differential |  |  | VIN | 180 |  | 850 | mV | 2 |
| Input Differential Impedance |  |  | ZIN | 90 | 100 | 110 | Ω |  |
| TX Disable | Disable |  |  | 2.0 |  | Vcc | V |  |
|  | Enable |  |  | 0 |  | 0.8 | V |  |
| TX Fault | Fault |  |  | 2.0 |  | Vcc | V |  |
|  | Normal |  |  | 0 |  | 0.8 | V |  |
| Receiver |  |  |  |  |  |  |  |  |
| Centre Wavelength |  |  | λc | 1260 |  | 1600 | nm |  |
| Receiver Sensitivity |  |  |  |  |  | -23 | dBm | 3 |
| Receiver Overload |  |  |  | -7 |  |  | dBm | 3 |
| LOS De-Assert |  |  | LOSD |  |  | -24 | dBm |  |
| LOS Assert |  |  | LOSA | -35 |  |  | dBm |  |
| LOS Hysteresis |  |  |  | 0.5 |  | 4 | dB |  |
| Data Output Swing Differential |  |  | Vout | 300 |  | 900 | mV | 4 |
| LOS |  |  | High | 2.0 |  | Vcc | V |  |
|  |  |  | Low |  |  | 0.8 | V |  |

Notes:

1. The optical power is launched into SMF.

2. PECL input, internally AC-coupled and terminated.

3. Measured with a PRBS 231-1 test pattern @10312Mbps, BER ≤1×10-12.

4. Internally AC-coupled.

- Pin Descriptions

| Pin | Signal Name | Description | Plug Seq. | Notes |
| --- | --- | --- | --- | --- |
| 1 | VEET | Transmitter Ground | 1 |  |
| 2 | TX FAULT | Transmitter Fault Indication | 3 | Note 1 |
| 3 | TX DISABLE | Transmitter Disable | 3 | Note 2 |
| 4 | SDA | SDA Serial Data Signal | 3 |  |
| 5 | SCL | SCL Serial Clock Signal | 3 |  |
| 6 | MOD_ABS | Module Absent. Grounded within the module | 3 |  |
| 7 | RS0 | Not Connected | 3 |  |
| 8 | LOS | Loss of Signal | 3 | Note 3 |
| 9 | RS1 | Not Connected | 3 |  |
| 10 | VEER | Receiver ground | 1 |  |
| 11 | VEER | Receiver ground | 1 |  |
| 12 | RD- | Inv. Received Data Out | 3 | Note 4 |
| 13 | RD+ | Received Data Out | 3 | Note 4 |
| 14 | VEER | Receiver ground | 1 |  |
| 15 | VCCR | Receiver Power Supply | 2 |  |
| 16 | VCCT | Transmitter Power Supply | 2 |  |
| 17 | VEET | Transmitter Ground | 1 |  |
| 18 | TD+ | Transmit Data In | 3 | Note 5 |
| 19 | TD- | Inv. Transmit Data In | 3 | Note 5 |
| 20 | VEET | Transmitter Ground | 1 |  |

Notes:

Plug Seq.: Pin engagement sequence during hot plugging.

1) TX Fault is an open collector output, which should be pulled up with a 4.7k~10kΩ resistor on the host board to a voltage between 2.0V and Vcc+0.3V. Logic 0 indicates normal operation; Logic 1 indicates a laser fault of some kind. In the low state, the output will be pulled to less than 0.8V.

2) Laser output disabled on TDIS >2.0V or open, enabled on TDIS <0.8V.

3) LOS is open collector output.  Should be pulled up with 4.7k~10kΩ on host board to a voltage between 2.0V and 3.6V. Logic 0 indicates normal operation; logic 1 indicates loss of signal.

4) RD-/+: These are the differential receiver outputs. They are internally AC-coupled 100 differential lines which should be terminated with 100Ω (differential) at the user SERDES.

5) TD-/+: These are the differential transmitter inputs. They are internally AC-coupled, differential lines with 100Ω differential termination inside the module.

Ordering information

| Part Number | Product Description |
| --- | --- |
| HD-SFP+/10G-ZR | 1550nm, 10.3Gbps, LC, 80km, 0°C~+70°C, with DDM |
| HD-SFP+/10G-ZR-I | 1550nm, 10.3Gbps, LC, 80km, -40°C~+85°C, with DDM |