1.25G SFP Transceiver  Duplex LC  20Km

HD-S3112-20LCD

- Features

  - Hot-pluggable SFP footprint

  - Duplex LC fiber connector

  - Transmission distance up to 20Km on 9/125µm SMF

  - Built-in DDMI functions

  - Comply with SFP MSA .

  - Single +3.3V power supply

  - RoHS6 compliant (lead free)

  - Operating case temperature:

Commercial :   0 to +70°C

Industrial :     -40 to +85°C

  - Comply with Ethernet Switches from a lot of widely used brands .

- Applications

  - Gigabit Ethernet

  - 1X Fiber Channel

  - Switch to Switch interface

  - Router/Server interface

  - Other optical transmission systems

- Optical Parameters

| Parameter | Symbol | Min. | Typical | Max. |  | Unit |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter Section: |  |  |  |  |  |  |  |  |
| Centre Wavelength | λc | 1260 | 1310 | 1360 | nm |  |  |  |
| Spectral Width (RMS) | σ |  |  | 3 | nm |  |  |  |
| Average Optical Power | Pavg | -9 |  | -3 | dBm |  | 1 |  |
| Laser Off Power | Poff | - | - | -45 | dBm |  |  |  |
| Extinction Ratio | ER | 9 | - | - | dB |  |  |  |
| Relative Intensity Noise | Rin | - | - | -128 | dB/Hz |  | 12dB reflection |  |
| Transmitter jitter | UIp-p |  | - | 0.1 |  |  | 3 |  |
| Optical Return Loss Tolerance |  | - | - | 12 | dB |  |  |  |
| Output Optical Eye | Compliant with IEEE802.3 z |  |  |  |  |  | 4 |  |
| Receiver Section: |  |  |  |  |  |  |  |  |
| Receiver Center Wavelength | λr | 1260 |  | 1620 | nm |  |  |  |
| Receiver Sensitivity in Average Power | Sen | -23 |  | -3 | dBm |  | 5 |  |
| Los Assert | LosA | -35 | - | - | dBm |  |  |  |
| Los Dessert | LosD | - | - | -24 | dBm |  |  |  |
| Los Hysteresis | LosH | 0.5 | - | - | dB |  |  |  |
| Overload | Pin | - | - | -3 | dBm |  | 5 |  |
| Receiver Reflectance |  | - | - | -12 | dB |  |  |  |
| Receiver power (damage) |  | - | - | 0 | dBm |  |  |  |

Notes:

1. The optical power is launched into 9/125µm SMF.

2. Unfiltered, measured with a PRBS 223-1 test pattern @1.25Gbps

3. Meet the specified maximum output jitter requirements if the specified maximum input jitter is present.

4. Measured with a PRBS 223-1 test pattern @1.25Gbps.

5. Measured with a PRBS 223 -1 test pattern @1.25Gbps, worst-case extinction ratio, BER ≤1×10-10

Pin Definitions

| Pin | Signal Name | Description | Plug Seq | Notes |
| --- | --- | --- | --- | --- |
| 1 | VeeT | Transmitter Ground | 1 |  |
| 2 | TX FAULT | Transmitter Fault Indication | 3 | Note 1 |
| 3 | TX Disale | Transmitter Disable | 3 | Note 2 |
| 4 | MOD_DEF(2) | I2CSerial Data Signal SDA | 3 | Note 3 |
| 5 | MOD_DEF(1) | I2CSerial Clock Signal SCL | 3 | Note 3 |
| 6 | MOD_DEF(0) | Grounded in Module | 3 | Note 3 |
| 7 | Rate Select | Not Connected | 3 |  |
| 8 | LOS | Receiver Loss of Signal Indicator (Active high, open-drain) | 3 | Note 4 |
| 9 | VeeR | Receiver Ground | 1 |  |
| 10 | VeeR | Receiver Ground | 1 |  |
| 11 | VeeR | Receiver Ground | 1 |  |
| 12 | RD- | Inv. Received Data Out | 3 | Note 5 |
| 13 | RD+ | Received Data Out | 3 | Note 5 |
| 14 | VeeR | Receiver Ground | 1 |  |
| 15 | VccR | Receiver Power Supply | 2 |  |
| 16 | VccT | Transmitter Power Supply | 2 |  |
| 17 | VeeT | Transmitter Ground | 1 |  |
| 18 | TD+ | Transmit Data In | 3 | Note 6 |
| 19 | TD- | Inv. Transmit Data In | 3 | Note 6 |
| 20 | VeeT | Transmitter Ground | 1 | Note 2 |

Ordering information

| Part Number | Product Description |
| --- | --- |
| HD-S3112-20LCD | 1310nm, 1.25Gbps, 20KM, Duplex LC SFP Transceiver, 0ºC ~ +70ºC , With DDM. |
| HD-S3112-20LCD-I | 1310nm, 1.25Gbps, 20KM, Duplex LC SFP Transceiver ,-40ºC ~ +85ºC, With DDM. |